Santa Fe County, NM

Wednesday, July 1, 2026

Exhibit 150A. Sustainable Land Development Code

CHAPTER 9. COMMUNITY DISTRICTS

§ 9.8. LA CIENEGA AND LA CIENEGUILLA COMMUNITY
DISTRICT OVERLAY.

9.8.1 Purpose and Intent. The provisions of the La Cienega and La Cieneguilla Community District Overlay (LCLCCD) are intended to implement and be consistent with the land use goals, objectives, policies, and strategies of the La Cienega and La Cieneguilla Community Plan and the Sustainable Growth Management Plan (SGMP). The LCLCCD is designed to ensure compatibility among various land uses, encourage compact development, protect scenic features and environmentally sensitive areas, enhance rural development patterns, accommodate a variety of lifestyles, delineate historic community spaces and conserve water resources while accommodating the anticipated natural growth of the community. The LCLCCD includes standards and dimensions based on historic development patterns and incentives to facilitate compact development where appropriate in conjunction with conservation easements that preserve agricultural land and open space.

9.8.2 Sustainable Design Standards. The development standards of Chapter 7 of this Code shall be applicable to all development, except as otherwise specified herein.

9.8.2.1 Water Supply, Wastewater and Water Conservation. Standards shall be regulated as identified in Chapter 7 of this Code with the following exceptions:

1. Residential Connection to County Utility Water System.

a. All new lots created as part of residential land divisions and subdivisions shall be required to connect to the Santa Fe County Water System when said system is extended to within 200 feet of the property line of a lot, unless that lot has previously connected to a community water system, provided that adequate capacity exists in the system and that water taps are available. This requirement will be applied with the following conditions:

i. If the water system is already in place and capable of providing service or if the County can provide an estimated time of completion of six (6) months or less, connection to the system will be required immediately upon the time of service capability within the system. The requirement for connection shall be duly noted on the survey plat, and the property owner shall agree to bonding or financial guarantee to ensure connection within six (6) months, prior to final approval.

ii. If the County cannot provide an estimated time for water line completion and capacity for service of six (6) months or less, the new land division will be granted a 2-year grace period from the time the water line is actually installed and taps are available before the agreement to connect to the system will be effective. The requirement for connection shall be duly noted on the survey plat prior to final approval.

iii. If connection to a county or community water system is not possible at the time of land division for new residential development of two or more lots of 2.5 acres or less, all lots will be required to use shared wells where new lots are adjoining, providing adequate water is available for both lots. The requirement to connect to the County or community water system will still be in effect and the property owners will be granted a 2-year grace period from the time the water line is actually installed and has adequate capacity, before the connection to the system will be required. The requirement for connection shall be duly noted on the survey plat prior to final approval.

2. Nonresidential Connection to County Utility Water System.

a. All new nonresidential development shall be required to connect to the Santa Fe County Water System when said system is extended to within 200 feet of the property line, unless the development has previously connected to a community water system, provided that adequate capacity exists in the system and that water taps are available.

i. If connection to the County water system is not possible, the development must limit water consumption to .35 acre-feet per year.

(a) If the water system is already in place and capable of providing service or if the County can provide an estimated time of completion of six (6) months or less, connection to the system will be required immediately upon the time of service capability within the system. The requirement for connection shall be duly noted on the survey plat and the property owner shall agree to bonding or financial guarantee to ensure connection within six (6) months prior to final approval.

(b) If the County cannot provide an estimated time for water line completion and capacity for service of six (6) months or less, the new land division will be granted a 2-year grace period from the time the water line is actually installed and taps are available before connection to the system will be required. The requirement for connection shall be duly noted on the survey plat prior to final approval.

9.8.2.2 Terrain Management. Standards shall be regulated as identified in Chapter 7 of this Code with the following exceptions:

1. Special Protection of Riparian Areas. Development Standards in Riparian Buffers. The standards and criteria of Chapter 7 of this Code shall apply to any portion of a development or, as appropriate, to any land disturbance within a riparian buffer with the following exceptions:

a. Intent. The restoration and enhancement of riparian areas and wetlands in the Planning Area is recognized as a potentially valuable resource management practice, so long as such projects are designed and implemented to maximize benefits to the ecosystem and water resources while avoiding harm to the health, safety and welfare of residents, including but not limited to loss of private property and public infrastructure. Therefore, all projects which propose restoration, enhancement and/or new construction of riparian areas or wetlands must demonstrate that the project will have a beneficial or neutral impact on existing water resources. No project shall be permitted that negatively impacts existing water rights.

b. Agriculture as defined in this Code is permitted within Riparian Corridors.

Figure 9-8-1: Riparian Corridors

| Table 9-8-1: Riparian Buffer Corridors |  |  |  |
| --- | --- | --- | --- |
| (A) Use | (B) Stream Side Zone | (C) Managed Use Zone | (D) Upland Zone |
| Trails, greenways, open space, parks or other similar public recreational uses and private recreational uses that do not require the use of fertilizers, pesticides, or extensive use of fences or walls. | P | P | P |
| Outdoor horticulture, forestry, wildlife sanctuary, and other similar agricultural and related uses not enumerated elsewhere in this table that do not require land-disturbing activities, or use of pesticides or extensive use of fences or walls. | C | C | C |
| Pastures or plant nurseries that do not require land-disturbing activities or use of pesticides, or extensive use of fences or walls. | C | C | C |
| Gardens, play areas, recreational uses and other similar uses that do not require the use of pesticides for routine maintenance | N | P | P |
| Lawns, golf course fairways, play fields, and other areas that may require the use of fertilizers or pesticides. | N | N | N |
| Sidewalks streets, bridges, and other similar transportation facilities. | C | C | C |
| Accessory land-disturbing activities ordinarily associated with a single- or two-family dwelling, such as utility service lines, and similar uses. | N | P | P |
| Public maintenance of streets, bridges, other similar transportation facilities and/or public utility and storm drainage facilities. | P | P | P |
| Detention/retention basin and associated infrastructure. | N | C | C |
| Lakes, ponds, and associated infrastructure, such as dams, spillways, riser pipes, and stilling basins, which are located outside of the regulatory floodplain. | C | C | C |
| Stream and riparian area restoration and maintenance. | P | P | P |
| New construction or enhancement of riparian area. | C | C | C |

P = the activity is permitted as of right; N = the activity is prohibited; and C = the activity is permitted only upon approval of a conditional use permit or a subdivision application.

9.8.3 Establishment of Zoning Districts. The zoning standards of Chapter 8 of this Code shall be applicable to all development, except as otherwise specified herein.

9.8.3.1 Generally. The LCLCCD modifies the underlying base zoning districts in order to address special siting, use, and compatibility issues requiring regulations that supplement or supplant those found in the base zoning districts of this Code. If an overlay zone regulation conflicts with any standard of the underlying zone, the standard of the overlay zone shall govern.

9.8.3.2 Density Requirements on Lots with Multiple Zoning Designations. If a single land parcel and/or subdivision lies within two separate zoning districts, the density of dwelling units per acre shall be calculated based upon the exact proportion of each district in which the parcel lies.

9.8.3.3 Density Bonus. Density Bonuses are not allowed in the LCLCCD boundary until further review is available.

9.8.3.4 Lot Coverage. Standards shall be regulated as identified in Chapter 7 of this Code with the following exceptions:

1. Residential Uses. The maximum lot coverage for residential uses is 20%.

2. Nonresidential Uses.

a. The maximum lot coverage for nonresidential uses is 60%.

b. The maximum aggregate building size is 50,000 square feet.

c. The maximum individual establishment size is 15,000 square feet.

9.8.3.5 Shooting ranges are prohibited throughout the LCLCCD planning area.

9.8.3.6 Base Zoning Districts. Base zoning districts approved for use in the La Cienega and La Cieneguilla Community District are listed in Table 9-8-2.

| Table 9-8-2: LCLCCD Community District Base Zoning |  |
| --- | --- |
| A/R | Agriculture/Ranching |
| RUR-F | Rural Fringe |
| RUR-R | Rural Residential |
| RES-F | Residential Fringe |
| RES-E | Residential Estate |
| RES-C | Residential Community |
| TC | Traditional Community |
| CN | Commercial Neighborhood |
| PI | Public/Institutional |
| PDD | Planned Development |

1. LCLCCD Agriculture/Ranching (LCLCCD A/R); Purpose. The purpose of this district is to designate areas suitable for agricultural, ranching and residential uses, and to prevent encroachment of incompatible uses and the premature conversion of agricultural and ranch lands to nonagricultural uses. Uses in the A/R district are limited to agricultural, ranch, residential and other compatible uses. This designation reflects areas whose present use is agricultural, such as grazing or dry land farming. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-3.

| Table 9-8-3: Dimensional Standards LCLCCD A/R (Agriculture/Ranching) |  |
| --- | --- |
| Zoning District | LCLCCD A/R |
| Density (# of acres per dwelling unit) | 160 |
| Lot width (minimum, feet) | 400 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) hay or animal barn, silo | 50 |
| Height (maximum, feet) all other structures | 36 |

2. LCLCCD Rural Fringe (LCLCCD RUR-F); Purpose. The purpose of this district is to designate areas suitable for a combination of estate-type residential development, agricultural uses and other compatible uses. The RUR-F designation provides an intermediate step in development density between typical open space and agricultural/ranching lands and primarily residential (low density) parcels. This zone also serves to protect agricultural and environmental areas that are inappropriate for more intense development due to their sensitivity. The RUR-F zone accommodates primarily large lot residential, ecotourism, equestrian uses and renewable resource-based activities, seeking a balance between conservation, environmental protection and reasonable opportunity for development. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-4.

| Table 9-8-4: Dimensional Standards LCLCCD RUR-F (Rural Fringe) |  |
| --- | --- |
| Zoning District | LCLCCD RUR-F |
| Density (# of acres per dwelling unit) | 20 |
| Lot width (minimum, feet) | 100 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) hay or animal barn, silo | 50 |

| Table 9-8-4: Dimensional Standards LCLCCD RUR-F (Rural Fringe) |  |
| --- | --- |
| Zoning District | LCLCCD RUR-F |
| Height (maximum, feet) all other structures | 36 |

3. LCLCCD Rural Residential (LCLCCD RUR-R); Purpose. The purpose of this district is to provide for the development of single-family homes on large lots, either individually or as part of rural subdivisions; to preserve the scenic and rural character of the County; to provide consolidated open space and agricultural lands; and to recognize the desirability of carrying on compatible agricultural operations and home developments in areas near the fringes of urban development while avoiding unreasonable restrictions on farming or ranching operations. Uses that support rural character of the broader area shall be allowed including agricultural production, small-scale renewable energy production, home-based businesses, bed and breakfasts, agro-tourism, equestrian and boarding facilities, farmers’ markets and produce stands. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-5.

| Table 9-8-5: Dimensional Standards LCLCCD RUR-R (Rural Residential) |  |
| --- | --- |
| Zoning District | LCLCCD RUR-R |
| Density (# of acres per dwelling unit) | 10 |
| Lot width (minimum, feet) | 100 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 24 |

4. LCLCCD Residential Fringe (LCLCCD RES-F); Purpose. The purpose of this district is to designate areas suitable for a combination of estate-type residential development, smaller-scale agricultural uses, ranchettes and other compatible uses. The RES-F district provides an intermediate step in single-family residential development between open space and/or agricultural/ranching lands, and typically suburban residential densities. The RES-F district may be comprised of a variety of residential lot sizes, clustered housing and community open space and can include limited agricultural use accessory to residential uses. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-6.

| Table 9-8-6: Dimensional Standards LCLCCD RES-F (Residential Fringe) |  |
| --- | --- |
| Zoning District | LCLCCD RES-F |
| Density (# of acres per dwelling unit) | 5 |
| Lot width (minimum, feet) | 100 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 24 |

5. LCLCCD Residential Estate (LCLCCD RES-E); Purpose. The purpose of the Residential Estate (RES-E) district is to designate areas suitable for a combination of large-lot and suburban-type residential development, ranchettes and other compatible uses. The RES-E district supports single-family homes on medium sized lots consistent with contemporary community development. Generally this district applies to low to medium density residential development in established neighborhoods (lands that are already committed to residential uses and have been subdivided for a specific development) and undeveloped or underdeveloped areas with a moderate to high development suitability. This category may include limited agricultural use accessory to residential uses. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-7.

| Table 9-8-7: Dimensional Standards LCLCCD RES-E (Residential Estate) |  |
| --- | --- |
| Zoning District | LCLCCD RES-E |
| Density (# of acres per dwelling unit) | 2.5 |
| Frontage (minimum, feet) | 100 |
| Lot width (minimum, feet) | 100 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 24 |

6. LCLCCD Residential Community (LCLCCD RES-C); Purpose. Generally this district applies to existing medium to higher density residential development in established neighborhoods (lands that are already committed to residential uses and have been subdivided for a specific development). The Residential Community designation applies to only a contiguous residential area in La Cieneguilla that is almost completely built out on existing 1-acre lots created in the late 70’s. No additional areas are proposed to have this zoning designation within the La Cienega and La Cieneguilla Planning Boundary.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-8.

| Table 9-8-8: Dimensional Standards LCLCCD RES-C (Residential Community) |  |
| --- | --- |
| Zoning District | LCLCCD RES-C |
| Density (# of acres per dwelling unit) | 1 |
| Frontage (minimum, feet) | 100 |
| Lot width (minimum, feet) | 100 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 24 |

7. LCLCCD Traditional Community (LCLCCD TC); Purpose. The purpose of this district is to designate areas suitable for residential, small-scale commercial and agricultural uses consistent with the existing development patterns of traditional communities. The TC district accommodates traditional community patterns, preserves historic and cultural landscapes, and protects agricultural uses, including agriculture found in traditional communities with acequia systems, from encroachment by development. Density bonuses and transfers of development rights may be utilized to achieve the purposes of the district. Density transfers and clustered development shall be allowed in order to support continued farming and/or ranching activities, conserve open space or protect scenic features and environmentally sensitive areas.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-9.

| Table 9-8-9: Dimensional Standards LCLCCD TC (Traditional Community) |  |
| --- | --- |
| Zoning District | LCLCCD TC |
| Density (# of acres per dwelling unit) | 0.75 |
| Frontage (minimum, feet) | 50 |
| Lot width (minimum, feet) | 50 |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 24 |
| Maximum building size (commercial) | 2,500 sq. ft. |

8. LCLCCD Commercial Neighborhood (LCLCCD CN); Purpose. The purpose of this district is to allow for low-intensity convenience retail and personal services, as well as office uses that are intended to serve and are in close proximity to individual residential neighborhoods.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-10.

| Table 9-8-10: Dimensional Standards LCLCCD CN (Commercial Neighborhood) |  |
| --- | --- |
| CN Zoning District | LCLCCD CN |
| Density | 2.5 |
| Frontage (minimum, feet) | 50 |
| Lot width (minimum, feet) | n/a |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 28 |
| Lot coverage (maximum, percent) | 60 |
| Maximum building size (aggregate) | 50,000 |
| Maximum size of individual establishments (sq. ft.) | 15,000* |

*Establishment size may be increased up to 30,000 square feet with the issuance of a conditional use permit.

9. LCLCCD Public Institutional (LCLCCD PI); Purpose. The purpose of this district is to accommodate governmental, educational, and nonprofit or institutional uses, including public or community parks and recreation facilities, and public, market shops, offices, animal production, livestock, poultry, nonprofit, and institutional residential uses, but excluding any such uses of an extensive heavy industrial character.

a. Use Regulations. Uses permitted, conditional and prohibited as identified in Chapter 8 and Appendix B of this Code with exceptions identified on LCLCCD Use Table [Table 9.8.15].

i. Where the purpose of the use is primarily that of a museum, temporary accessory uses shall be permitted, which shall include the erection of temporary structures and tents for shelter, the holding or [of] retreats, the offering of restaurants with or without incidental consumption of alcoholic beverages, the offering of a stand-alone store or shop.

b. Dimensional Standards. The dimensional standards shall be as identified in Chapter 8 of this Code except as prescribed in Table 9-8-11.

| Table 9-8-11: Dimensional Standards LCLCCD PI (Public/Institutional) |  |
| --- | --- |
| PI Zoning District | LCLCCD PI |
| Frontage (minimum, feet) | 40 |
| Lot width (minimum, feet) | n/a |
| Lot width (maximum, feet) | n/a |
| Height (maximum, feet) | 48 |
| Lot coverage (maximum, percent) | 60 |

i. Density. The density for the PI zoning district is dependent on the surrounding zoning districts. For the Las Golondrinas property the section of the property that was surveyed as part of the Traditional Community will have one dwelling unit per 0.75 acre density and the remaining portion of the parcel and southern parcel will have a one dwelling unit per 2.5 acre density.

10. LCLCCD PDD (Planned Development District); Purpose. PDDs identified on the initial zoning map may be built out in accordance with their approved master plans including density and uses.

9.8.3.7 Density Transfers.

1. Density transfers should be used to protect community assets including but not limited to wetlands, open spaces, springs, watercourses, riparian areas, agricultural land, acequias, traditional community centers, archaeological sites, historical and cultural sites, and multigenerational family compounds. When density transfers result in higher site densities, such development shall be clustered and sited in a manner to fit the topography, and existing rural character of La Cienega and La Cieneguilla, instead of in an urban grid pattern. A land density transfer program allows the transfer of all or part of the permitted

density on a parcel to another parcel or other locations on the same parcel. This may include concepts such as family compounds or density transfers to protect agricultural, open space or other land protection or preferred development patterns.

a. Open Space Land Protection through Density Transfers or other types of Easements. Open spaces set aside for density transfer or other easements for the protection of community assets, as described above, should interconnect to similar sites or potential sites on adjacent properties whenever possible. Easements shall clearly delineate the open space areas and no build areas as well as document any permitted uses in such areas.

b. Lot Coverage Calculations on Density Transfers or other types of Perpetual Easements. Whenever density transfer is used to create open space which is protected in perpetuity by easement or other legal means, and new lots are created, lot coverage shall be calculated based on the total acreage of the project rather than for each individual development lot and shall be apportioned to the density transfer lots by means of building envelopes on the approved final plat.

9.8.4 Supplemental Zoning Standards. Standards shall be regulated as identified in Chapter 10 of this Code with the following exceptions:

9.8.4.1 Home Occupations. The purpose of the Home Occupations in the LCLCCD is to support economic development opportunities that are not disruptive to the residential character of the area or significantly interfere with the use of adjacent residential properties. Home Occupations shall be as regulated in Chapter 10 of this Code except as prescribed in Table 9-8-12.

| Table 9-8-12: Home Occupations |  |  |  |
| --- | --- | --- | --- |
|  | No Impact | Low Impact | Medium Impact |
| Permit type | Business Registration | Development Permit | Conditional Use Permit |
| Non-resident employees (max) | 1 | 3 | 4 |
| Area used for business (maximum) | 25% of heated square footage | 35% of heated square footage | 50% of heated square footage |
| Accessory building storage | 100 SF | 600 SF | 1,500 SF |
| Appointments/patron visits (max/day) | 0 | 4 | 12* |
| Business traffic | none | see Section 10.6.5 | see Section 10.6.5 |
| Signage | not permitted | see Section 7.9.4.3 [sic] | see Section 7.9.4.3 [sic] |
| Parking and access | Resident and employee only | see Section 10.6.5 | see Section 10.6.5 |
| Heavy Equipment | None | Up to 2 | 3-6 |

*Small-scale retail establishments, such as arts and crafts stores, restaurants or galleries may be permitted to have 30 vehicle visits per day and no more than 10 vehicle visits per hour.

9.8.4.2 Accessory Structures. Standards shall be regulated as identified in Chapter 10 of this Code with the following exceptions:

1. Accessory Structures over 2000 square feet are required to obtain a Conditional Use permit.

| Table 9-8-13: Increased Units Allowed per TDR in the County and LCLCCD |  |  |
| --- | --- | --- |
| Use | Additional Unit per TDR in the County | Additional Unit per TDR in LCLCCD |
| Residential | 3 | 4 |
| Nonresidential | 5,000 sf | 6,000 sf |

9.8.7 Use Table. Uses permitted in the base zones and planned development zoning districts are shown in the LCLCCD Use Table 9-8-15. All uses are designated as permitted, accessory, or conditional, as further explained in Table 9-8-14. Accessory uses may be subject to specific regulations as provided in Chapter 9, 10, and conditional uses are subject to the conditional use permit standards provided in Chapter 4. In addition, uses may be subject to modification by the overlay zoning regulations included in this chapter.

| Table 9-8-14: Use Table Labels |  |
| --- | --- |
| P | Permitted Use: The letter “P” indicates that the listed use is permitted by right within the zoning district. Permitted uses are subject to all other applicable standards of the SLDC. |
| A | Accessory Use: The letter “A” indicates that the listed use is permitted only where it is accessory to a use that is permitted or conditionally approved for that district. Accessory uses shall be clearly incidental and subordinate to the principal use and located on the same tract or lot as the principal use. |
| C | Conditional Use: The letter “C” indicates that the listed use is permitted within the zoning district only after review and approval of a Conditional Use Permit in accordance with Chapter 4. |
| DCI | Development of Countywide Impact: The letters “DCI” indicate that the listed use is permitted within the zoning district only after review and approval as a Development of Countywide Impact. |
| X | Prohibited Use: The letter “X” indicates that the use is not permitted within the district. |

The Use Matrix also includes Function, Activity and Structure Codes in accordance with the Land Based Classification System.

Uses not specifically enumerated. When a proposed use is not specifically listed in the use matrix, the Administrator may determine that the use is materially similar to a listed use if:

The proposed use is listed as within the same structure or function classification as the use specifically enumerated in the use matrix as determined by the Land-Based Classification Standards (LBCS) of the American Planning Association (APA).; or

If the use cannot be located within one of the LBCS classifications, the Administrator shall refer to the most recent manual of the North American Industry Classification System (NAICS). The proposed use shall be considered materially similar if it falls within the same industry classification of the NAICS manual. A proposed use shall be treated in the same manner as the listed use to which it is materially similar. For example, if the proposed use is materially similar to a permitted use, this proposed use shall also be a permitted use.

[View the Editable Version of the Use Table here]